- کاربردهای کنونی پیل سوختی میکروبی

اولین وسیله آزمایشی که با استفاده از تکنولوژی پیل‌سوختی میکروبی مورد بهره‌برداری قرار گرفت و در سال 2008 گزارش شد [43] یک شناور هواشناسی با قابلیت اندازه‌گیری دما، فشار، رطوبت نسبی دمای آب و انتقال دیتا از طریق امواج رادیویی توسط بنتنیک ام اف سی ساخته شد.

بنتیک ام اف سی با استفاده از اکسیداسیون ترکیبات عالی موجود در رسوبات لایه‌های آب اقیانوسی اقدام به تولید برق نموده است. الکترون‌ها در اثر متابولیسم طبیعی میکرواورگانیزم‌های رسوبات دریایی تولید می‌شوند. بنتیک ام اف سی به هیچگونه میکرواورگانیزم اکسیژنی اضافی نیازمند نیست. دو مدل مختلف از نمونه‌های بنتیک ام اف سی ساخته شد و مورد استفاده قرار گرفت اولی یک نمونه 230 کیلوگرمی با حجم 1.3 که می‌توانست معادل 16 باتری آلکالین در سال برق تولید کند. این طراحی به مقدار کمی آلودگی نیازمند است و می‌تواند توسط یک نفر نصب گردد. این نمونه جهت تأمین برق دستگاه اندازه‌گیری هواشناسی بدون نیاز به تعویض باتری طراحی و ساخته شده است. دستگاه‌های بنتیک ام اف سی برای چندین سال بدون کاهش در توان خروجی به کار خود ادامه دادند. نویسنده تخمین میزند که بنتیک ام اف سی توانایی تولید نامحدود برق در اعماق دریا را با هزینه یک باطری اسیدی برای یک سال را داراست.

محققان روش‌هایی را جهت افزایش توان بنتیک ام اف سی در نظر گرفته‌اند. درحالی‌که توان الکتریکی تولیدی از متابولیسم میکرواورگانیزم‌های موجود در رسوبات نشات می‌گیرد. شاید یکی از محدودیت‌ها در تولید سطوح بالاتری  انرژی محدودیت در میکرواورگانیزم‌های در دسترس باشد. اضافه کردن ترکیبات آلی غیرقابل‌حل که به‌آرامی مورد استفاده قرار می‌گیرند، نظیر سلولز یا کتیتن به رسوبات باعث افزایش توان تولیدی می‌گردد [44] [45].

این لایه‌ها به دلیل ارزان و در دسترس بودن منابع جذابی محسوب می‌شوند. افزایش چگالی توان و مدت‌زمان آن متأثر از اندازه ذرات لایه‌های غیرقابل‌حل آلی هستند. توان‌های بالاتر تولیدی در طول مدت‌زمان کوتاه‌تر باکاهش اندازه ذرات کتیتن قابل دستیابی هستند. افزایش توان تولیدی در بنتیک ام اف سی برای یک مدت‌زمان محدود قابل دستیابی است اما قابل ادامه یافتن برای زمان طولانی نخواهد بود مگر اینکه مواد آلی به‌صورت مداوم جایگزین گردند. همچنین می‌توان بنتیک ام اف سی را در زیر مزارع پرورش ماهی و پساب‌های کشاورزی که حاوی منابع غنی آلی هستند نصب نمود. یکی از محدودیت‌های پیل‌سوختی میکروبی هزینه مواد آن به‌طور مثال لایه نفیونی آن است. درحال حاضر تحقیقاتی جهت تولید پیل‌های سوختی میکروبی ارزان با استفاده از ظروف سفالی جهت استفاده در هند در جریان است [46]. در این پیل‌ها از ظروف سفالی، فولاد زنگ نزن به‌عنوان کاتد و صفحات گرافیتی به‌عنوان آند استفاده می‌گردد.

کاربردهای بالقوه پیل سوختی میکروبی

استفاده از یک آند به‌عنوان پذیرنده الکترون با استفاده از باکتری‌ها کاربردهای بالقوه فراوانی را برای این پیل‌ها ایجاد کرده است. قابل ذکر است در حال حاضر بسیاری از این کاربردها امکان استفاده را نداشته و نیازمند مطالعات بیشتر در این زمینه‌ها هستند. یکی از زمینه‌های مورد توجه در پیل‌های سوختی میکروبی تولید توان با استفاده از پساب‌ها همراه اکسیداسیون مواد آلی و غیر آلی می‌باشد. تحقیقات نشان می‌دهد هر ماده‌ای که توانایی تجزیه توسط باکتری‌ها را داشته باشد می‌تواند جهت تولید توان مورد استفاده قرار گیرد. رنج گسترده‌ای از مواد جهت استفاده در پیل‌های سوختی میکروبی مورد استفاده قرار می‌گیرند. در این سیستم‌ها مقدار کمی بایومس نیز تولید می‌گردد که باعث کاهش انرژی مورد نیاز جهت طی فرایند هوازی و بی هوازی می‌گردد.

استفاده از پیل‌های سوختی میکروبی جهت تصفیه پساب‌ها با مشکلاتی از قبیل تبدیل از مقیاس آزمایشگاهی به مقیاس بزرگ و نیز سرعت کند واکنش‌ها روبروست. توانایی پیل‌های سوختی میکروبی در تجزیه رنج گسترده‌ای از آلودگی‌ها می‌تواند بسیار باارزش‌تر از تولید الکتریسیته با استفاده از آن‌ها باشد بخصوص در مواردی جهت پاک‌سازی محیط‌زیست مورد بهره‌برداری قرار گیرد. مشخص شده که برخی از گونه‌های باکتری‌ها توانایی تجزیه محصولات نفتی و ترکیبات سدیمی در آب‌های زیرزمینی را دارا هستند. اکسیداسیون آلودگی‌ها در ارتباط با کاهش اتم‌های آهن هستند. فرایندهای اکسایش و کاهش می‌توانند با استفاده از ا افزودن اتم‌های آهن یا ناقل‌های الکترونی سرعت پیدا کنند. به‌طور معمول آلودگی‌ها به دلیل عدم وجود پذیرنده‌های الکترونی در محیط باقی می‌مانند و امکان اضافه کردن آن‌ها به محیط نیز وجود ندارد. کشت باکتری‌های کاهنده فلزی توانایی اکسیداسیون بنزوات و تلوئن را با استفاده از اضافه کردن یک پذیرنده الکترونی دارا هستند. قرار دادن یک الکترود درون آلودگی‌های هیدروکربنی خاک می‌تواند باعث افزایش سرعت تجزیه تلوئن، بنزوات و نفتالین گردد. [47]

نمونه‌ای از یک پیل سوختی آزمایشی [47]

استفاده از یک الکترود به‌عنوان پذیرنده الکترونی در خاک بسیار جذاب است، بطوریکه میکروب‌ها می‌توانند بر روی بستر آند گرافیتی به تجزیه آلودگی‌ها بپردازند. در این حالت الکترودها می‌توانند به‌عنوان یک چاه الکترونی برای مدت‌زمان طولانی به تجزیه مواد مضر برای طبیعت کمک کنند. در این شرایط الکترون‌های که توسط میکروب‌ها به‌صورت جریان الکتریکی تولید شده است بی‌ارتباط با افزایش سرعت فرایند زیستی به نظر می‌رسد. به‌این‌ترتیب تحقیقات نشان می‌دهد که پیل‌های سوختی میکروبی می‌توانند موانع تخمیر در طی فرایند تجزیه زیست‌توده‌های سلولوزی را برطرف کنند [48]. حذف موانع تخمیر باعث افزایش سرعت تجزیه مواد می‌گردد درحالی‌که مقدار کمی انرژی نیز تولید می‌نماید. یک کاربرد غیرمعمول از پیل‌های سوختی میکروبی تأمین توان ایمپلنت‌های پزشکی با استفاده از اکسیژن و گلوکز موجود در خون است. یک پیل سوختی میکروبی ایمپلنتی می‌تواند انرژی را به‌صورت پیوسته بدون نیاز عمل جراحی جهت تعویض باتری ایمپلنت را تأمین نمایید. پیل‌های سوختی میکروبی زیستی از کاتالیست‌های فلزات نجیب و کربن فعال برای تولید انرژی از گلوکز خون در آزمایشگاه ساخته شده‌اند. همچنین پیل‌های سوختی با کاتالیست های آنزیمی تحت شرایط فیزیولوژیکی آزمایش شده‌اند اما نیازمند توسعه‌های بیشتر جهت رسیدن به قابلیت اطمینان هستند. همچنین استفاده از گلبول‌های سفید به‌عنوان منبع الکترونی آند مورد توجه قرار گرفته‌اند.

شماتیک یک تصفیه‌خانه MFC [48]

تحقیقات آزمایشگاهی صورت گرفته بر روی پیل میکروبی

اثر غلظت سابستریت بر عملکرد پیل سوختی پیوسته

تحقیقاتی که توسط مهروران فر و همکاران صورت گرفته [49] اثر تغییرات غلظت سابستریت ها بر روی عملکرد پیل سوختی پیوسته را به‌صورت زیر به نمایش می‌گذارد:

اثر تغییر غلطت سابستریت ها بر دانسیته توان

منحنی پلاریزاسیون در غلظت‌های مختلف سابستریت ها

تحقیقاتی که توسط Min Hea Kim [50] در سال 2009 صورت گرفته است اثر مقادیرمختلف غلظت لاکتات بر پیل سوختی میکروبی با دومحفظه آند همزمان مورد بررسی قرار گرفته است که شکل آن در زیر قابل روئیت است.

اثر غلظت سابستربر چگالی جریان پیل [50]

در تحقیقات آزمایشگاهی که توسط قریشی و همکاران صورت گرفته است [51] اثر تغییر غلظت گلوکوز بر جریان و ولتاژ تولیدی توسط پیل میکروبی و نیز توان خروجی پیل موردبررسی قرار گرفته است به‌طوری‌که افزایش غلظت گلوکوز تااندازه‌ای باعث افزایش توان خروجی و پس‌ازآن باعث کاهش توان به‌دست‌آمده خواهد شد که شکل آن در زیر قابل مشاهده است.

تغییرات پلاریزاسیون پیل میکروبی با تغییر غلظت گلوکوز

همچنین در تحقیق دیگری که توسط کونگ و همکاران [52] انجام شده منحنی پلاریزاسیون پیل میکروبی به ازای غلظت‌های مختلف گلوکوز مطابق زیر گزارش شده است:

تغییرات پلاریزاسیون پیل میکروبی با تغییر غلظت گلوکوز

تغییرات توان تولیدی پیل میکروبی با تغییر غلظت گلوکوز

اثر pH بر روی عملکرد پیل میکروبی

تحقیقی که توسط سارا مدنی و همکاران بر روی پیل میکروبی صورت گرفته اثر غلظت بایومس و pH بر پیل میکروبی را مورد بررسی قرار داده که قسمتی از نتایج تجربی حاصل به‌صورت زیر قابل مشاهده است [53].

اثر pH بر توان خروجی پیل

اثر سابستر بر رشد باکتری‌ها

تحقیقات صورت گرفته توسط پینک چاک و همکاران [54]  میزان رشد باکتری شوانولا را به ازای سابسترهای مختلف و اثر غلظت لاکتات بر سرعت رشد آن‌ها به‌صورت تجربی مورد بررسی قرار داده است که نتایج آن در شکل زیر قابل روئیت است :

اثر غلظت سابستر بر رشد باکتری‌ها

اثر انواع سابستر بر رشد باکتری شوانولا

اثر پارامترهای محفظه آند

محفظه آند مهم‌ترین بخش یک پیل سوختی است بطوریکه شرایط اولیه‌ای که در آن زیست‌توده‌ای تجزیه شده و آماده تولید جریان الکتریکی می‌گردد در این محفظه رخ می‌دهد. این محفظه با سابستر ها، مواد واسط (اختیاری) میکرواورگانیزم‌ها و الکترود آند پر شده است. این مسئله که عوامل متعددی در عملکرد پیل سوختی اثرگذارند به‌خوبی روشن است. در تحقیقات گوناگون اثرات مواد مختلف بکار رفته در محفظه آند بر روی عملکرد پیل سوختی میکروبی بررسی شده است که نتایج آن توسط Mostafa Rahimnejad و همکاران [55] جمع‌آوری و ارائه شده است :

اثر پارامترهای مختلف محفظه آند بر عملکرد پیل سوختی میکروبی

اثر پارامترهای محفظه کاتد

پروتون‌های تولیدی در سمت آند با عبور از غشا به سمت کاتد مهاجرت کرده تا در آنجا جریان الکتریکی را تکمیل نمایند، الکترود کاتد به‌عنوان کاتالیست نقش مهمی در سرعت واکنش کاهش در کاتد دارد درنتیجه انتخاب نوع الکترود کاتد می‌تواند بر عملکرد پیل اثرگذار باشد که تغییرات نوع کاتد بر روی توان تولیدی در تحقیقات گوناگون بررسی و توسط مصطفی رحیم نژاد و همکاران [55] ارائه شده است :

اثر پارامترهای مختلف محفظه کاتد بر عملکرد پیل سوختی میکروبی

مدل‌سازی‌های صورت گرفته برای پیل میکروبی

یک مدل‌سازی یک‌بعدی توسط اولیوریا و همکاران [56] برای پیل میکروبی با استفاده از معادلات باتلر- ولمر، نرنست و تافل صورت گرفته که در منحنی پلاریزاسیون پیل با داده‌های تجربی مورد مقایسه قرار گرفته است که خروجی تحقیق در زیر قابل مشاهده است:

مدل‌سازی پلاریزاسیون پیل میکروبی به‌صورت ریاضی

مدل‌سازی توان خروجی پیل میکروبی به‌صورت ریاضی

در این تحقیق همچنین تغییرات Overpotential درازای مقادیر مختلف جریان به‌صورت زیر محاسبه شده است:

تغییرات بیش پتانسیل پیل میکروبی

همچنین تحقیقات دیگری برای شبیه‌سازی ریاضی پیل میکروبی توسط پیرسپینتو [57] صورت گرفته که با تولید مدل ریاضی نسبت به بهینه‌سازی خروجی توان اقدام کرده پس‌ازآن با استفاده از مدل‌سازی صورت گرفته نسبت به ساخت آزمایشگاهی پیل مبادرت شده است.

در تحقیقات دیگری که توسط پیسیوریانو و همکاران انجام‌شده تغییرات بایوفیلم در پیل میکروبی مورد شبیه‌سازی کامپیوتری صورت گرفته است.

تحقیق دیگری توسط مانوهار و همکاران [58]صورت پذیرفته که در آن نسبت به شبیه‌سازی پیل میکروبی با میکروب شوانولا وماده غذایی لاکتات اقدام شده است در ایشبیه‌سازیزی با استفاده از مقادیر بدست آمده ازداده های آزمایشگاهی نسبت بشبیه‌سازیزی  تغییرات افت‌های پیل میکروبی و همچنین محاسبه چگالی جریان تبادلی پیل مذکور اقدام شده است.

درتحقیقات صورت گرفته توسط یینگژی زنگا و همکاران [59] یک مدل ریاضی براشبیه‌سازیزی پیل میکروبی با ماده غذایی استاانجام‌شدهده که تغییرات pH ، Overpotential ، نرخ واکنش، غلظت و. . . مورشبیه‌سازی قرار گرفته که قسمتی از نتایج حاصله درشکل 2-18 قابل مشاهده است :

اثرپارامتر های مختلف پیل میکروبی بر عملکرد آن

همچنین در تحقیق دیگری که توسط لیژائوو همکاران صورت گرفته [60] افت‌های پیل سوختی میکروبی شبیه‌سازی شده است که نتیجه حاصل از مدل‌سازی ریاضی آن در شکل 2-19 قابل مشاهده می‌باشد:

تغییرات افت پتانسیل در پیل میکروبی

شماتیک تغییرات افت پتانسیل پیل میکروبی با افزایش جریان خروجی

در جدول صفحه بعد کارهای تحقیقاتی صورت گرفته که در بالا معرفی شده‌اند،به‌صورت خلاصه فهرست می‌گردد:

خلاصه برخی ازتحقیقات صورت گرفته بر روی پیل سوختی میکروبی

| پارامترهای مورد بررسی |  |  |  |  |  |  |  |  |  |  |  |  |  |  | بررسی میکروسکوپی |  | بررسی ماکروسکوپی |  |  | آزمایشگاهی |  |  |  |  | شبیه‌سازی |  | عنوان مقاله |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| پلاریزاسیون |  | باکتری |  | الکترود |  |  | pH |  | بایوفیلم |  | اغتشاش |  | اکسیژن |  |  |  | سابستر |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ✓ |  |  |  |  |  |  |  |  |  |  |  |  | ✓ |  |  |  |  |  | ✓ |  | ✓ |  |  |  |  | بررسی اثر غلظت سابستریت در عملکردپیل‌های سوختی‌میکروبی پیوسته [49] |  |  |  |  |
| ✓ |  |  |  |  |  |  |  |  |  |  |  |  | ✓ |  |  |  |  |  | ✓ |  | ✓ |  |  |  |  | An Analysis of Anaerobic Dual-Anode Chambered[50] |  |  |  |  |
| ✓ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ✓ |  | ✓ |  |  |  |  | Effect of type and concentration of substrate on power generation in a dual chambered microbial fuel cell [51] |  |  |  |  |
| ✓ |  |  |  |  |  |  |  |  |  |  |  |  | ✓ |  |  |  |  |  | ✓ |  | ✓ |  |  |  |  | Performance Investigation of Batch Mode Microbial Fuel Cells Fed With High Concentration of Glucose [52] |  |  |  |  |
| ✓ |  |  |  |  |  | ✓ |  |  |  |  |  |  | ✓ |  |  |  |  |  | ✓ |  | ✓ |  |  |  |  | Optimization of the performance of a double-chamber microbial fuel cell through factorial design of experiments and response surface methodology[53] |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | ✓ |  | ✓ |  |  |  |  |  | ✓ |  | ✓ |  |  |  |  | Constraint-Based Model of Shewanella oneidensis MR-1 Metabolism: A Tool for Data Analysis and Hypothesis Generation[54] |  |  |  |  |
| ✓ |  | ✓ |  | ✓ |  |  |  |  |  |  |  |  | ✓ |  |  |  |  |  | ✓ |  |  |  |  |  |  | Microbial fuel cell as new technology for bioelectricity generation: A review[54] |  |  |  |  |
| ✓ |  |  |  |  |  | ✓ |  |  |  |  |  |  | ✓ |  |  |  |  |  | ✓ |  |  |  | ✓ |  |  | A 1D mathematical model for a microbial fuel cell[56] |  |  |  |  |
| ✓ |  |  |  |  |  |  |  |  |  |  |  |  | ✓ |  |  |  |  |  | ✓ |  |  |  | ✓ |  |  | DYNAMIC MODELLING AND OPTIMISATION OF MICROBIAL FUEL CELLS AND MICROBIAL ELECTROLYSIS CELLS [57] |  |  |  |  |
| ✓ |  | ✓ |  |  |  |  | ✓ |  |  |  | ✓ |  |  |  |  |  |  |  | ✓ |  |  |  | ✓ |  |  | Mathematical model for microbial fuel cells with anodic biofilms and anaerobic digestion[61] |  |  |  |  |
| ✓ |  | ✓ |  | ✓ |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ✓ |  |  |  | ✓ |  |  | The polarization behavior of the anode in a microbial fuel cell[58] |  |  |  |  |
| ✓ |  |  |  | ✓ |  | ✓ |  |  |  |  |  |  | ✓ |  |  |  |  |  | ✓ |  |  |  | ✓ |  |  | Modelling and simulation of two-chamber microbial fuel cell[59] |  |  |  |  |
| ✓ |  |  |  | ✓ |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ✓ |  |  |  | ✓ |  |  | MODELING OF POLARIZATION LOSSES OF A MICROBIAL FUEL CELL[60] |  |  |  |  |
|  |  |  |  |  |  |  |  |  | ✓ |  | ✓ |  |  |  |  |  |  |  | ✓ |  |  |  | ✓ |  |  | Modeling of a Microbial Fuel Cell[62] |  |  |  |  |
| ✓ |  |  |  |  |  |  |  |  | ✓ |  | ✓ |  | ✓ |  |  | ✓ |  |  | ✓ |  |  |  | ✓ |  |  | تحقیقات این پایان نامه |  |  |  |  |

| پارامترهای مورد بررسی; نگاه میکروسکوپی; نگاه ماکروسکوپی; آزمایش تجربی; شبیه‌سازی; نام مقاله; نویسنده. پلاریزاسیون; باکتری; الکترود; pH; بایوفیلم; اغتشاش; اکسیژن; سابستر. ✓; ✓; ✓; ✓; بررسی اثر غلظت سابستریت در عملکردپیلهای سوختی‌میکروبی پیوسته; مهروران فروهمکاران [49]. ✓; ✓; ✓; ✓; An Analysis of Anaerobic Dual-Anode Chambered; مین هیا کیم [50]. ✓; ✓; ✓; ✓; Effect of type and concentration of substrate on power generation in a dual chambered microbial fuel cell; قریشی و همکاران [51]. ✓; ✓; ✓; ✓; Performance Investigation of Batch Mode Microbial Fuel Cells Fed With High Concentration of Glucose; کونگ و همکاران [52]. ✓; ✓; ✓; ✓; ✓; Optimization of the performance of a double-chamber microbial fuel cell through factorial design of experiments and response surface methodology; سارا مدنی و همکاران [53]. ✓; ✓; ✓; ✓; Constraint-Based Model of Shewanella oneidensis MR-1 Metabolism: A Tool for Data Analysis and Hypothesis Generation; پینک چاک و همکاران [54]. ✓; ✓; ✓; ✓; ✓; Microbial fuel cell as new technology for bioelectricity generation: A review; مصطفی رحیم نژاد و همکاران [55]. ✓; ✓; ✓; ✓; ✓; A 1D mathematical model for a microbial fuel cell; اولیوریا و همکاران [56]. ✓; ✓; ✓; ✓; DYNAMIC MODELLING AND OPTIMISATION OF MICROBIAL FUEL CELLS AND MICROBIAL ELECTROLYSIS CELLS; پیرسپینتو [57]. ✓; ✓; ✓; ✓; ✓; ✓; Mathematical model for microbial fuel cells with anodic biofilms and anaerobic digestion; پیسیوریانو و همکاران [61]. ✓; ✓; ✓; ✓; ✓; The polarization behavior of the anode in a microbial fuel cell; مانوهار و همکاران [58]. ✓; ✓; ✓; ✓; ✓; ✓; Modelling and simulation of two-chamber microbial fuel cell; یینگژی زنگا و همکاران [59]. ✓; ✓; ✓; ✓; MODELING OF POLARIZATION LOSSES OF A MICROBIAL FUEL CELL; لیژائوو همکاران [60]. ✓; ✓; ✓; ✓; Modeling of a Microbial Fuel Cell; مایکل الکساندر کالدر [62]. ✓; ✓; ✓; ✓; ✓; ✓; ✓; شبیه‌سازی پیل سوختی میکروبی و بررسی اثر متغییر های ورودی بر روی عملکرد آن; نوشتار پیش‌رو. |  |
| --- | --- |

نوآوری تحقیقات پیشِ رو

در تحقیقاتی که تاکنون بر روی پیل‌های سوختی میکروبی صورت گرفته به‌طورکلی یا به‌صورت آزمایشگاهی خروجی پیل به ازای پارامترهای عملکردی مورد ارزیابی قرار گرفته و یا شبیه‌سازی‌های بر مبنای مدل‌های ریاضی بوده است که مشخصه‌های خروجی پیل به‌صورت ماکروسکوپی موردتوجه قرار گرفته‌اند اما در این تحقیقات اثر پارامترهای میکروسکوپی با استفاده از شبیه‌سازی در فلوئنت بررسی شده است به‌طوری‌که نرخ نفوذ مواد در الکترود آند وکاتد تغییرات غلظت اکسیژن و سابستر، نرخ محلی واکنش کاتد و... مورد توجه قرار گرفته، و با استفاده از شبیه‌سازی مشخصه‌های مذکور نسبت به تعیین جریان محدودکننده پیل میکروبی اقدام شده و در نهایت از آن برای تعیین منحنی پلاریزاسیون پیل استفاده شده است. لذا تحقیقات پیش رو در جهت شناخت مکانیزم‌های درونی پیل میکروبی گام برمی دارد.

منابع و مراجع

| [1] | امین احمد‌پور,منصور کلباسی, “بررسی عملکرد پیل سوختی در تولید انرژی‌های پاک,” در چهارمین کنفرانس بین المللی رویکردهای نوین در نگهداشت انرژی, 1390. |
| --- | --- |
| [2] | محمدتقی بهرامی‌پور,سیدمحمود ربیعی; مجتبی جعفریان, “بررسی انواع پیل‌های سوختی با نگرش مقایسه‌ای و کاربرد آن‌ها در صنایع نظامی,” در اولین همایش انرژی‌های نو پاک, 1391. |
| [3] | M. Johan, "Recent developments in microbial fuel cells: A review Article," In Journal of scientific and industrial research, vol. 23, pp. 48-53, 2010. |
| [4] | B. Kim, H. Park, H. Kim, G. Kim, I. Chang, J. Lee and N. Phung, "Enrichment of microbial community generating electricity using a fuel-cell-type electrochemical cell," App. Microbiol and Biotechnol, vol. 63, pp. 672-681, 2004. |
| [5] | P. e. a. Aelterman, "Continuous electricity generation at high voltages and currents using stacked microbial fuel cells. 40(10):3388–3394," Environ Sci Technol, vol. 40, no. 10, p. 3388–3394, 2006. |
| [6] | J. J. G. G. K. M. K. H. C. B. Chang I, "Continuous determination of biochemical oxygen demand using microbial fuel cell type biosensor," Biosens, vol. 19, pp. 607-613, 2004. |
| [7] | G. S. a. Z. T.A, "Polymer electrolyte fuel cell," Adv. Electrochem.Sci, vol. 5, pp. 195-301, 1997. |
| [8] | D. Bond, D. Holmes, L. Tender and D. Lovley, "Electrode-reducing microorganisms that harvest energy from marine sediments," Science, vol. 298, pp. 483-485, 2002. |
| [9] | K. Rabaey, N. Boon, S. Siciliano and M. Verhaege, "Biofuel cells select for microbial consortia that self-mediate electron transfer," Appl. Environ. Microbiol, vol. 70, pp. 5373-5382, 2004. |
| [10] | S. Freguia, K. Rabaey, Z. Yuan and J. Keller, "Syntrophic processes drive the conversion of glucose in microbial fuel cell anodes," Environ. Sci. Technol, vol. 42, pp. 7937-7943, 2008. |
| [11] | H. Lee, P. Parameswaran, A. Kato-Marcus, C. Torres and B. Rittmann, "Evaluation of energy-conversion efficiencies in microbial fuel cells (MFCs) utilizing fermentable and non-fermentable substrates," Water Res, vol. 42, pp. 1501-1510, 2008. |
| [12] | M. Lanthier, K. Gregory and D. Lovley, "Growth with high planktonic biomass in Shewanella oneidensis fuel cells," FEMS Microbiol. Lett, vol. 278, pp. 29-35, 2008. |
| [13] | D. Bond and D. Lovley, "Evidence for Involvement of an Electron Shuttle in Electricity Generation by Geothrix fermentans," Appl. Environ. Microbiol, vol. 71, pp. 2186-2189, 2005. |
| [14] | D. Lovley, "Bug juice: harvesting electricity with microorganisms," Nature Rev. Microbiol, Vols. 497-508, p. 4, 2006. |
| [15] | K. Rabaey, G. Lissens, S. Siciliano and W. Verstraete, "A microbial fuel cell cabable lf converting glucose to electricity at high rate and efficiency," Biotech. Lett, vol. 21, pp. 1531-1535, 2003. |
| [16] | S. Cheng, H. Liu and B. Logan, "Power densities using different cathode catalysts (Pt and CoTMPP) and polymer binders (Nafion and PTFE) in single chamber microbial fuel cells," Env. Sci. Tech, vol. 40, pp. 364-369, 2006. |
| [17] | F. Zhao, F. Harnisch, U. Schroder, F. Scholz, P. Bogdanoff and I. Herrmann, "Application of pyrolysed iron(II) phthalocyanine and CoTMPP based oxygen reduction catalysts as cathode materials in microbial fuel cells," Electrochem. Comm, vol. 7, pp. 1405-1410, 2005. |
| [18] | B. Logan, "Exoelectrogenic bacteria that power microbial fuel cells," Nat. Rev. Microbiol, vol. 7, pp. 375-381, 2009. |
| [19] | K. Rabaey, N. Boon, M. Hofte and W. Verstraete, "Microbial phenazine production enhances electron transfer in biofuel cells," Environ. Sci. Technol, vol. 39, pp. 3401-3408, 2005. |
| [20] | H. Richter, K. Nevin, H. Jia, D. Lowy, D. Lovley and L. Tender, "Cyclic voltammetry of biofilms of wild type and mutant Geobacter sulfurreducens on fuel cell anodes indicates possible roles of OmcB, OmcZ, type IV pili, and protons in extracellular electron transfer," Energ. Environ. Sci, vol. 2, p. 506–516, 2009. |
| [21] | S. Srikanth, E. Marsili, M. Flickinger and D. Bond, ". Electrochemical characterization of Geobacter sulfurreducens cells immobilized on graphite paper electrodes," Biotechnol. Bioeng, vol. 99, p. 1065–1073, 2008. |
| [22] | G. Reguera, K. McCarthy, T. Mehta, J. Nicoll, M. Tuominen and D. Lovley, "Extracellular electron transfer via microbial nanowires," Nature 2005, vol. 435, p. 1098–1101, 2005. |
| [23] | C. Myers and J. Myers, ". Localization of cytochromes to the outer membrane of anaerobically grown Shewanella putrefaciens MR-1," J. Bacteriol, vol. 174, p. 3429–3438, 1992. |
| [24] | L. Tender, C. Reimers, H. Stecher, D. Holmes, D. Bond, D. Lowy, K. Pilobello, S. Fertig and D. Lovley, "Harnessing microbially generated power on the seafloor," Nat. Biotechnol, vol. 20, pp. 821-825, 2002. |
| [25] | D. Bond and D. Lovley, "Electricity production by Geobacter sulfurreducens attached to electrodes," Appl. Environ. Microbiol, vol. 69, pp. 1548-1555, 2003. |
| [26] | D. Holmes, D. Bond, R. O'Neil, C. Reimers, L. Tender and D. Lovley, "Microbial communities associated with electrodes harvesting electricity from a variety of aquatic sediments," Microbial Ecol, vol. 48, pp. 178-190, 2004. |
| [27] | K. Nevin, H. Richter, S. Covalla, J. Johnson, T. Woodard, A. Orloff, H. Jia, M. Zhang and D. Lovley, "Power output and columbic efficiencies from biofilms of Geobacter sulfurreducens comparable to mixed community microbial fuel cells," Environ. Microbiol, vol. 10, pp. 2505-2514, 2008. |
| [28] | D. Lovley, K. Nevin and C. D. Harwood, "Electricity production with electricigens. In Bioenergy: Microbial Contributions to Alternative Fuels," ASM Press, vol. 28, pp. 298-306, 2008. |
| [29] | C. Picioreanu, I. Head, K. Katuri, M. van Loosdrecht and K. Scott, "A computational model for biofilm-based microbial fuel cells," Water Res, vol. 41, pp. 2921-2940, 2007. |
| [30] | H. Yi, K. Nevin, B. Kim, A. Franks, A. Klimes, L. Tender and D. Lovley, "Selection of a variant of Geobacter sulfurreducens with enhanced capacity for current production in microbial fuel cells," Biosens. Bioelectron, vol. 24, pp. 3498-3503, 2008. |
| [31] | A. Kato-Marcus, C. Torres and B. Rittmann, "Conduction-based modeling of the biofilm anode of a microbial fuel cell," Biotechnol. Bioeng, vol. 98, pp. 1171-1182, 2007. |
| [32] | K. Nevin, B. Kim, R. Glaven, J. Johnson, T. Woodard, B. Methe, R. DiDonato, S. Covalla, A. Franks, A. Liu and D. Lovley, "Anode biofilm transcriptomics reveals outer surface components essential for high density current production in Geobacter sulfurreducens fuel cells," PLoS ONE, vol. 4, p. 5628, 2009. |
| [33] | C. Torres, H. Lee and B. Rittmann, "Carbonate species as OH- carriers for decreasing the pH gradient between cathode and anode in biological fuel cells," Environ. Sci. Technol, vol. 42, p. 8773–8777, 2008. |
| [34] | C. Torres, A. Kato Marcus and B. Rittmann, "Proton transport inside the biofilm limits electrical current generation by anode-respiring bacteria," Biotechnol. Bioeng, vol. 100, pp. 872-881, 2008. |
| [35] | H. Lee, C. Torres and B. Rittmann, "Effects of Substrate Diffusion and Anode Potential on Kinetic Parameters for Anode-Respiring Bacteria," Environ. Scienc. Technol, vol. 43, pp. 7571-7577, 2009. |
| [36] | A. Franks, K. Nevin, R. Glaven and D. Lovley, "Microtoming Coupled to Microarray Analysis to Evaluate the Spatial Metabolic Status of Geobacter sulfurreducens Biofilms," ISME J, vol. 1038, p. 137, 2010. |
| [37] | B. Logan, B. Hamelers, R. Rozendal, U. Schroder, J. Keller, S. Freguia, P. Aelterman, W. Verstraete and K. Rabaey, "Microbial fuel cells: Methodology and technology," Environ. Sci. Technol, vol. 40, p. 5181–5192, 2006. |
| [38] | H. Rismani-Yazdi, S. Carver, A. Christy and O. Tuovinen, "Cathodic limitations in microbial fuel cells: An overview," J. Power Sourc, vol. 180, p. 683–694, 2008. |
| [39] | G. Gil, I. Chang, B. Kim, M. Kim, J. Jang, H. Park and H. Kim, "Operational parameters affecting the performannce of a mediator-less microbial fuel cell," Biosens. Bioelectron, vol. 18, pp. 327-334, 2003. |
| [40] | D. Park and J. Zeikus, "Improved fuel cell and electrode designs for producing electricity from microbial degradation," Biotechnol. Bioeng, vol. 81, p. 348–355, 2003. |
| [41] | K. Gregory, D. Bond and D. Lovley, "Graphite electrodes as electron donors for anaerobic respiration," Env. Microbiol, vol. 6, pp. 596-604, 2004. |
| [42] | H. Tran, D. Kim, S. Oh, K. Rasool, D. Park, R. Zhang and D. Ahn, ". Nitrifying biocathode enables effective electricity generation and sustainable wastewater treatment with microbial fuel cell," Water Sci. Technol, vol. 59, p. 1803–1808, 2009. |
| [43] | "The first demonstration of a microbial fuel cell as a viable power supply: Powering a meteorological buoy," Power Sourc, vol. 179, pp. 571-575, 2008. |
| [44] | F. Rezaei, T. Richard and B. Logan, "Enzymatic hydrolysis of cellulose coupled with electricity generation in a microbial fuel cell," Biotechnol. Bioeng, vol. 110, p. 1163–1169, 2008. |
| [45] | F. Rezaei, T. Richard, R. Brennan and B. Logan, "Substrate-enhanced microbial fuel cells for improved remote power generation from sediment-based systems," Environ. Sci.Technol, vol. 41, pp. 4053-4058, 2007. |
| [46] | M. Behera, P. Jana and M. Ghangrekar, "Performance evaluation of low cost microbial fuel cell fabricated using earthen pot with biotic and abiotic cathode," Bioresour. Technol, vol. 101, pp. 1183-1189, 2009. |
| [47] | T. Zhang, S. Gannon, K. Nevin, A. Franks and D. Lovley, "Stimulating the anaerobic degradation of aromatic hydrocarbons in contaminatedsediments by providing an electrode as the electron acceptor," Environm. Microbiol. Rep, vol. 73, pp. 348-427, 2010. |
| [48] | A. Borole, J. Mielenz, T. Vishnivetskaya and C. Hamilton, "Controlling accumulation of fermentation inhibitors in biorefinery recycle water using microbial fuel cells," Biotechnol. Biofuels, vol. 2, p. 7, 2009. |
| [49] | م. ا. ر. حدیثه مهروان‌فر, “بررسی اثر غلظت سابستریت در عملکردپیلهاي سوختی میکروبی پیوسته,” در چهارمین همایش ملی بیوانرژي ایران, تهران, 1392. |
| [50] | M. H. Kim, "An Analysis of Anaerobic Dual-Anode Chambered," Masters Theses, 8 2099. |
| [51] | A. Ghoreyshi, T.Jafary, G. Najafpour and F.Haghparast, "Effect of type and concentration of substrate on power generation in a dual chambered microbial fuel cell," in World newable energy Congeress 2011, linkoping sweden, 2011. |
| [52] | X. Kong, G. Yang and Y. Sun, "Performance Investigation of Batch Mode Microbial Fuel Cells Fed With High Concentration of Glucose," Biomwdical journal of scientifce & Technical Reshearch, vol. 3, no. 2, 2018. |
| [53] | S. Madani, R. Gheshlaghi, M. A. Mahdavi, M. Sobhani and A. Elkamel, "Optimization of the performance of a double-chamber microbial fuel cell through factorial design of experiments and response surface methodology," Fuel, vol. 150, pp. 434-440, 15Jun2015. |
| [54] | P. G. E., H. E. A., G. O. V. and I. J. De., "Constraint-Based Model of Shewanella oneidensis MR-1 Metabolism: A Tool for Data Analysis and Hypothesis Generation," PLoS Computational Biology, vol. 6, no. 6, 2010. |
| [55] | M. Rahimnejad, A. Adhami, S. Darvari, A. Zirepour and S.-E. Oh, "Microbial fuel cell as new technology for bioelectricity generation: A review," Alexandria Engineering Journal, pp. 36-42, 2015. |
| [56] | V. O. b, M. S. b, L. M. b and A. Pinto, "A 1D mathematical model for a microbial fuel cell," Energy, vol. 61, pp. 463-471, 2013. |
| [57] | R. P. Pinto, DYNAMIC MODELLING AND OPTIMISATION OF MICROBIAL FUEL CELLS AND MICROBIAL ELECTROLYSIS CELLS, UNIVERSITÉ DE MONTRÉAL, 2011. |
| [58] | A. K. Manohara and O. Bretschger, "The polarization behavior of the anode in a microbial fuel cell," Electrochimica Acta, vol. 53, p. 3508–3513, 2008. |
| [59] | Y. Zenga and Y. F. Choob, "Modelling and simulation of two-chamber microbial fuel cell," Journal of Power Sources, vol. 195, p. 79–89, 2010. |
| [60] | L. Zhao, J. Brouwer, J. Naviaux and A. Hochbaum, "MODELING OF POLARIZATION LOSSES OF A MICROBIAL FUEL CELL," in Proceedings of the ASME 2014 12th International Conference on Fuel Cell Science, Engineering and Technology, Bostone, 2014. |
| [61] | C. Picioreanu, K. P. Katuri, I. M. Head and M. C. M. v. L. a. K. Scott, "Mathematical model for microbial fuel cells with anodic biofilms and anaerobic digestion," Water Science & Technology—WST ¦, vol. 57.7, p. 965, 2008. |
| [62] | M. A. Calder, "Modeling of a Microbial Fuel Cell," Master of Science in Energy and Environment, August 2007. |
| [63] | D. Pinto, "Electronic transfer within a microbial fuel cell. Better understanding of Experimental and Structural Parameters at the Interface between Electro-active Bacteria and Carbon-based Electrodes," Doctoral thesis of Material Chemistry, 2 March 2017. |
| [64] | D. A. Elias, S. L. Tollaksen, D. W. Kennedy and H. M. Mottaz, "The infuence of cultivation methods on Shewanella oneidensis physiology and proteome expression," Arch Microbiol, vol. 189, p. 313–324, 2008. |
| [65] | Q. Wena, Y. Wua, D. C. a, L. Z. b and Q. Sun, "Electricity generation and modeling of microbial fuel cell from continuous beer brewery wastewater," Bioresource Technology, vol. 100, p. 4171–4175, 2009. |
| [66] | V. A. Sethuraman, S. Khan and J. S. Jur, "Measuring Oxygen, Carbon Monoxide and Hydrogen Sulfide Diffusion Coefficient and Solubility in Nafion Membranes," ELECTROCHIMICA ACTA, vol. 54, no. 27, pp. 6850-6860, 2009. |
| [67] | G. Kaur, "Chapter 2 Cell Voltages, Polarisations and Performances," in Solid Oxide Fuel Cell Componentsponents Interfacial Compatibility of SOFC Glass Seals, Springer International Publishing, 216, p. 408. |
| [68] | M. &eddy, Wastewater Engineering (TREATMENT DISPOSAL REUSE), McGRAW-HILL INTERNATIONAL EDITIONS. |
| [69] | Y. J. Tang, H. G. Martin and A. Deutschbauer, "Invariability of central metabolic flux distribution in Shewanella oneidensis MR-1 under environmental or genetic perturbations," Department of Energy, Environmental and Chemical Engineering, Washington University, St., Washington, 2017. |
| [70] | R. S. Renslow, B. Ahmed and J. R. Nuñez, "Modeling Substrate Utilization,Metabolite Production, and Uranium Immobilization in Shewanella oneidensis Biofilms," forintiers in Enviromental Science, vol. 5, p. 30, 2017. |
| [71] | A. Franks, K. Nevin, H. Jia, M. Izallalen, T. Woodard and D. Lovley, "Novel strategy for three-dimensional real-time imaging of microbial fuel cell communities: monitoring the inhibitory effects of proton accumulation within the anode biofilm," Energ. Environ. Sci., vol. 2, pp. 113-119, 2009. |
| [72] | S. e. a. Roller, "Electron-transfer coupling in microbial fuel cells: 1. Comparison of Redox mediator reduction rates and respiratory rates of bacteria," J Chem Technol, vol. 34B, pp. 3-12, 1984. |
| [73] | S. Kerzenmacher, J. DucrÈe, R. Zengerle and F. von Stetten, "Energy harvesting by implantable abiotically catalyzed glucose fuel cells," J. Power Sourc, vol. 182, pp. 1-17, 2008. |
| [74] | B. Logan and J. Regan, "Microbial fuel cells-challenges and applications," Environ. Sci. Technol, vol. 40, pp. 5172-5180, 2006. |

‌Benthic MFC

Substrate

Polarizasion

Butler-Vulmer

Nernest

Tafel